MCEdit V4.xx

User’s Manual

This manual may have different with the latest software.

20011.3

Chapter 1 Install and setup	4

1.1 Install the software	4

1.2 Connect computer and LED screen — Serial Port	4

1.3 Connect the computer and LED screen — Ethernet	5

1.4 Config firewall	5

1.5 Setup — Serial Communication	7

1.6 Setup — network communication	9

Chapter 2 Operation interface	12

2.1 Toolbar	12

2.2 Program list	14

2.3 Program item property	14

2.4 Simulating display	14

Chapter 3 Parameters setting	15

3.1 The hardware’s property of LED screen	15

3.2 LED screen’s size and color settings	17

Chapter 4 Making program	17

4.1 Add program	17

4.2 Program Playing property	18

4.3 Add word, excel file and image	18

4.4 Delete program item	19

Chapter 5 Sending program	20

5.1 Organize data	20

5.2 Transfer file(s)	20

Chapter 6	Design real-time screen	22

6.1 Background Words	23

6.2 Clock time item	23

6.3 Countdown	23

6.4 Temperature and Humidity	23

6.5 Serial Data	24

Chapter 7	Running info、update and security mechanism	25

7.1 Running Info	25

7.2 Update	25

7.3 Security Mechanism	27

Chapter 1 Install and setup

1.1 Install the software

MCEdit is green software, you just need double click MCEdit to lanuch the software.

1.2 Connect computer and LED screen — Serial Port

If using serial communication, the PC’s RS232 interface should be connected to LED screen’s RS232, or RS485 interface, the cable should exchange PIN2 and PIN3 at the two ends.

1.3 Connect the computer and LED screen — Ethernet

Besides serial communication, PC can also connect LED screens via Ethernet.

Below diagram is a RJ45 connector, and its first PIN and the lines’ order of 568A and 568B in EIA/ TIA standard.

Whether PC is directly connected to LED screen or PC is connected to LED screen through router, HUB, then the two terminals of the cable both should be 568B.

1.4 Config firewall

WindosXP firewall and the anti-virus program maybe will shut off some of PC’s program or functions, so it is very important to make the correct firewall settings to make the LED screen work well, the user can go to: [Start] -> [Setup] -> [Control Panel], double click “Windows firewall”, the Firewall settings interface will be as below:

Don’t check the box before “don’t allow exception” as in the below red ellipse.

Go to [exception], click [add program], add MCEdit.exe, check in the box before MCEdit.exe.

1.5 Setup — Serial Communication

CL3000 supports both serial and net communications.

The default serial port settings for both CL3000 are: COM1, the baud rate: 38400. To use serial port for communication following the blow steps:

Step 1:	Use the serial cable with PIN2 and Pin3 exchanged to connect the controller’s COM1 with PC’s serial port.

Step 2:	In the software, go to [tool] -> [Option], first to setup [LED], in the below dialog box, check the box before “Transfer data by serial port”.

Step 3:	In the software, go to [Tool] -> [Option] -> [PC serial port], setup PC’s serial port number, baud rate, Net Timeout (time delay) and Retry times, then click [ok].

Step 4:	In the menu, click [Run Time information] to get the running information (which means the communication has got through, it can use serial port to communicate and control the LED screen).

So far, it is ok to run CL3000 by sending/ receiving data through serial port.

It can setup the LED controller’s serial port through Ethernet while setting up the LED screen’s hardware, it also has 4 steps as below:

Step1:	In the menu, go to [tool] -> [option], setup [LED], don’t choose “transfer data by serial port”.

Step2:	Go to [LED Serial port], to setup controller’s serial port: [Data bit] is 8, [Stop bit] is 1, [Check bit] choose None, [Same Check] leave blank, mark the check before [Data Transfer], then click [OK].

Step3:	Go to [Tool] -> [Option] -> [LED], choose “Transfer data by serial port”.

Step4:	Go to [PC serial port], setup the same parameters as the settings in step2, and setup the [Time out] and [Retry times] according to the actual communication distance and the interference from the environment, then click [Ok].

1.6 Setup — network communication

First of all, it needs to get PC’s net parameters. In the PC’s local connection, right click property, to find TCP/ IP connection, click [Property], to find and remember the subnet mask parameters.

Then run MCEdit, in the menu, go to [Tool] -> [Option] -> [Parameter], the dialog box is as below:

When searching in the same net segment, then just clear the edit box, then click [Seek]; if searching across net segment, need input the LED’s IP address, then click [seek]. When the controller finds out the LED screens, it will show all the LED screen’s IP addresses in the window as in the above dialog box, then double click the IP line in need, to setup the net parameters.

For across net visiting, the router or the exchanger’s firewalls maybe have shut off the LED screen’s UDP port. In this case, it is needed to apply for UDP port from net management.

At first, input the local IP subnet mask code into the subnet mask box, setup the LED’s IP according to local IP address (PC’s IP) and subnet mask code.

Subnet mask and IP address both have 4 segments. If one segment of the subnet mask is 255, then the segment in the same location in LED’s IP will be as same as the one of local IP. In the above dialog box, the first 3 segments are all 255, so LED’s IP and local IP both have the same value in their first 3 segments, the first 3 segments are all 10.1.1, only the last segment is different. The Local IP’s last segment is 142, LED’s last segment is 127.

The name below the gateway in the above box, is used to name each LED screen, which is easy for users to remember.

Chapter 2 Operation interface

Run MCEdit, you can see the following GUI

2.1 Toolbar

The tool bar in the software has 4 kinds of buttons as below:

- For program manipulation: [New],[Load],[Save] and [Exit]

- [New]:	Clear the displaying programs, create a new programs in the file.

- [Load]:	Download the prepared programs.

- [Save]:	Save the current edited programs.

- [Exit]:	Exit SuperComEdit program.

- For program item manipulation: [add], [Delete]

- [Add]:	Create a new program, its interface will match the full screen, which can have multiple areas, and each area can have multiple items.

- [Delete]:	Delete one item in one area, and it is not able to delete one area, (but the user can move the area dividing line to get different sizes of areas).

- For editing manipulation: it includes [Cancel], [Add File], [Add Timer], [Add RTF], [Add Text], [Add Table]. Click on one editing button, then click on one displaying area, then can do the related editing manipulation.

- [Cancel]:	After clicking on one editing buttons, and the user wants to cancel this editing manipulation, just click on [Cancel], it will cancel the editing manipulation.

- [Add Image]:	Add one or more programs in one area, which can be images, text file, FLASH and word file.

- [Add Timer]:	Add clock timer in one area.

- [Add RTF]:	Add a RTF item in one area, can directly input the contents in the area, and right click the area, where can choose the font, color, and choose the text alignment.

- [Add Text]:	Add one text file in one area, similar as RTF file, but it cannot adjust the individual font and color, or part of the words, but it can enable the compacting layout of words.

- [Add Table]:	Add one table in one area.

4.	For the system manipulation: it includes [Preview], [Transfer] and [Runtime Info]

- [Preview]:	Display the programs on the monitor, when the LED screen is not connected, it can show the displaying effect.

- [Transfer]:	Transfer data.

- [Run Tim Info]:	Display the running information and update the screen’s player.

2.2 Program list

Program list is in the left part of the dialog box, has 3 levels:

The highest level is the program file name, if it is a new made program, then highest level is “new program’;

The second level is of areas, each program consists of one or multiple areas. Each area can display multiple items.

The third level are displaying items, which can be word, excel, BMP or JPG image, RTF, TEXT file and real-time screen items.

If one displaying item is a word file, double click it, it will automatically run the word program, and arrange the page layout according to the actual displaying size of the word file.

If one displaying item is an image, (BMP or JPG), double click on it, then it will automatically run the Windows paint program to edit the image.

If it is a real-time screen item, then double click on it, it will start the real-time screen editor.

2.3 Program item property

In the software, it can choose playing methods, moving speed and staying time and schedule. For the image file, it can choose [stretch] to make the picture fit the display window.

2.4 Simulating display

In the right part of the operating interface is an simulatting display.

For a RTF or TEXT file, it can be directly input and edited in the editing box; right click inside the box, it can set the font and its color. If it was a RTF file, then it is able to revise or change just part of the words’ font, sizes and colors.

If in one area, the item is a clock file, then this area will display the requested time;

If in one area, the item is a word file, then this area will display a word file.

Chapter 3 Parameters setting

Before the initial running of MCEdit, it has to setup the related parameters. Before setting up the parameter, it needs to power on the controller, and connect the communication cable. If just to practice making the displaying program in the software, then it is not necessary to power on the controller and connect the communication cable, in this situation, you can only setup the LED’s size (width x height).

In the MCEdit operating interface, go to [Tool] -> [option] to setup the parameters.

3.1 The hardware’s property of LED screen

The hardware parameter should be setup by professional engineers. If the parameter was incorrectly set, it may damage the LED screen. So it should be very careful to setup or revise the hardware parameters. Before accessing to parameters’ setting interface, the software will ask the user for the password, if the password was wrong, the dialog box will go back to LED parameters setting interface, and not allow revision or settings of the hardware parameters.

In the above dialog box, we can choose the Driver route type, the route type is described in the format: xx-Pyy [-nn-mm].

‘xx’ means scanning mode, it can be one of 16(1/16 scanning), 08(1/8 scanning) 04(1/4scanning), 02(1/2 scanning), or 01 (static mode), ‘yy’ indicates the row number which one signal port provides, normally it can be 02, 04, 08, 16; but very few can be 01. Referring to the different connection method between LED and IC pins, ‘nn’ and ‘mm’ can have multiple choices, ‘nn’ is for columns, ‘mm’ is for the rows. (Please refer to the lights panel in the website for more details)

Sometimes the sequences of the connected PINs are different, this will make the displaying patterns incorrect, so we can exchange the sequences for some columns and rows to get the correct displaying.

Scanning duty cycle is to adjust proportion of the high-level and low –level of the clock time. This can be very useful when the data signal need be transferred to the furthest end of the LED board.

3.2 LED screen’s size and color settings

LED setting is a very common setting. Before you make the displaying programs, the LED screen size should be setup, and when the LED screen is connected, a color bar will display.

Chapter 4 Making program

4.1 Add program

Click on [Add] button on the tool bar, the dialog box with templates of displaying layouts as below:

When users double click on one layout template, the schedule will be added one program with this layout. Choose the item, and click on [Property] button, to setup the brightness, power on/off time schedule and special effect.

4.2 Program Playing property

One program can consists of several (up to 3) areas, and each area can display multiple images, word file, RTF file, TEXT file and real-time screen.

Every program item in each area be played orderly and independently.

The user can drag the dividing line to adjust the areas size.

4.3 Add word, excel file and image

In the tool bar, click on [Add files] button, then click on area where the file need be placed, then to choose the file,

From the top left window of the above dialog box, to find the file location, and in the below column, show all the files and displaying programs, and double click on one program, then it will be added to the right window; or users just choose several programs together, and click ‘+’, then all the chosen programs can be added to the list in the right column.

On the top right area of the above interface, it can setup the files’ entering mode, moving speed and the staying time. For an image file, it can choose [stretch] to automatically fit the LED screen.

4.4 Delete program item

In the program list, click on the programs item, then click on [Delete], the programs item will be deleted.

Chapter 5 Sending program

5.1 Organize data

On the tool bar, when clicking on the [Transfer file(s)] button, MCEdit start to organize data. It will divide the Word, RTF and excel file into several files for displaying.

5.2 Transfer file(s)

After the data is rearranged, it will show the transfer management window.

In the left part of the interface are the well organized files (under PC C:\MC3000.tmp directory), and the user can just choose part of the files to transfer. If no file was chosen, then it will automatically transfer all the files.

Chapter 6	Design real-time screen

Click on the [Add timer] button in the tool bar, then click on the area where the time items need be placed, or double click on the clock items in the program, the clock displaying interface will be as below:

In the time displaying interface, the upper part is tool bar, the lower window is an imitating display.

The most left button on the tool bar is [Load background], and [Font]. “Load background’ can have an image as the background of the time displaying interface. “Font” is used to setup the displaying words, and its sizes and colors.

On the tool bar, [Font] button are followed with: [Text], [Year], [Month], [Date], [week], [Hour], [Minute], [Second], [Temperature], [Humidity], [Countdown], [Data from Serial port]. And the most right 3 buttons are [Delete], [Save] and [Exit]

6.1 Background Text

On the tool bar, click on [Text], then in the below imitating display, click on the place where the words should be input, and there is an editing box below the tool bar, where can input the words. These words will not be changed while displaying.

6.2 Clock time item

On the tool bar, click on [Year], [Month], [Date], [Week], [Hour], [Minute], or [Second], then click on the area in the imitating display where the clock items should be input. These clock items’ data are read from the LED controller.

Note: [hour] need choose the time zone. When you place [Hour] item in the displaying area, the system will automatically choose the time zone, but the user can also specify the time zone.

6.3 Countdown

On the tool bar, click on [Counter], then in the imitating display, click on the place where the countdown items need be input.

In the countdown property, the user can choose “Hide lead-zero’, ‘Bit number’, countdown direction.

6.4 Temperature and Humidity

On the tool bar, click on [Temperature] or [Humidity], and in the imitating display, click on the area where they need be input.

Note: it needs the hardware to support for the temperature and humidity displaying.

6.5 Serial Data

On the tool bar, click on [Data from serial port], then in the imitating display, click on the place where the serial data want to be input, the item will be added.

This item can choose to display the data from whichever serial port, which data from the serial port, and bit number.

It need setup the related the hardware of the serial port before the serial data can be displayed. In the menu, go to [Tool] -> [Option] where the parameters can be set.

Data displaying protocol: the leading code is ASII code “N”, it is a displaying string less than 8 groups of data, which ends with the Enter key(0dH+0aH) to skip to another line, each group is divided by “,”. Except for the ASCII code “0”-“9”, it still can use “+”, “-“, “.” and space.

Clock time checking protocol: the leading code is ASII code “T”, it is a 14 digits time data, which ends with the Enter key (0dH+0aH)

“T06091902101328“+0dH+0aH

The year has 2 digits, Sun is “00”.

Nx1,x2,… …, xn+[Enter]

Here N is the character ’N’, x1-xn are 8 groups of ASII digits, each group can only consists of space, decimal, ‘+’, ‘-‘ and the digit ‘0’- ‘9’. X1-xn is divided by ‘,’.

It don’t have to be 8 groups of data, it can be less than 8.

Chapter 7	Running info、update and security mechanism

7.1 Running Info

On the tool bar, click on [Runtime info], it will show the system’s running info as below:

The dialog box shows the software version, hardware version, SD card status, LED source supply status, the driver of current playing programs, and the current programs serial number, temperature, humidity and brightness sensors data, the controller’s clock time.

7.2 Update

LED screen’s playing software should agree with program’s making software (MCEdit), otherwise it cannot work well with each other.

While running the program, it reminds the box, which shows the conroller’s playing software is later than the program’s making software, in this case, PC need download the latest the version of program’s making software.

While running the program, it shows the dialog box, which means the program’s making software is later than the controller’s playing program, in this case, it just need go to [Runtime info], and when the [update] is active, then click it to update.

7.3 Security Mechanism

In the parameter setting interface, choose [Parameter], click on [Seek], all the LED screens in the LAN will show their MAC address and IP address in the list, double click on the LED which need set, then it will show the below dialog box:

After the IP address, subnet mask, and gateway are correctly setup, it can setup a password in order to prevent unauthorized person revising the related parameters. If the password is invoked, then before revision of the net parameters, the system will ask for and verify the password. If the password is incorrect, then the parameters cannot be set or revised.

As the LED is connected to the controller via LAN, any PC with MCEdit in the LAN can visit and control the LED screen. So In order to prevent the unauthorized person sending the data to the LED screen, in the MCEdit, the user can designate the PC’s IP who can visit the LED screen.

It can designate at most 16 IP addresses, or 16 passwords for the PCs who need visit the LED screen. After the settings of the password, it will need password to access to parameters setting and data sending and receiving. If the password was incorrect, the related settings and operations will not work.